# MAVLink Mission File — Acceptable Format & How to Send It From Your Laptop

## What Pixhawk/PX4 actually accepts

Pixhawk (running PX4) doesn't read a "file" directly — it accepts a mission uploaded over the MAVLink mission protocol (a MISSION_COUNT → MISSION_ITEM_INT exchange), the same protocol whether the source is a hand-written file, QGroundControl, or your project's own MAVSDK pipeline (app/mission/converter.py → app/mavlink/px4_client.py). "An acceptable mission file" really means a file format that a ground-station tool can parse and replay as that same protocol exchange.

Two file formats are effectively universal for this:

| Format | Extension | Style | Tooling |
| --- | --- | --- | --- |
| QGC WPL110 | .waypoints / .txt | Plain text, tab-separated, one line per mission item | MAVProxy (wp load/wp save), pymavlink, QGroundControl (legacy import) |
| QGroundControl Plan | .plan | JSON — mission + geofence + rally points together | QGroundControl (native/default format today) |

For hand-editing and scripting, QGC WPL110 is the simplest — one readable line per waypoint. That's the format below.

## Example acceptable mission file

A minimal, valid QGC WPL110 file matching this project's own demo mission shape (home → takeoff → 2 waypoints → return-to-launch):

QGC WPL 110

0	1	0	16	0	0	0	0	1.000000	2.000000	0.000000	1

1	0	3	22	0	0	0	0	0.000000	0.000000	30.000000	1

2	0	3	16	0	0	0	0	1.000000	2.000000	30.000000	1

3	0	3	16	0	0	0	0	1.010000	2.000000	30.000000	1

4	0	3	20	0	0	0	0	0.000000	0.000000	0.000000	1

Column order: index  current  frame  command  param1  param2  param3  param4  lat/x  lon/y  alt/z  autocontinue. Command 16 = MAV_CMD_NAV_WAYPOINT, 22 = MAV_CMD_NAV_TAKEOFF, 20 = MAV_CMD_NAV_RETURN_TO_LAUNCH. Frame 3 = MAV_FRAME_GLOBAL_RELATIVE_ALT (altitude relative to home).

The lat/lon values above (1.000000, 2.000000) are the exact placeholder coordinates this project's own _demo_cli() functions use — they are not real GPS coordinates. Replace them with your actual test-flight or campus coordinates before sending this to a real Pixhawk.

The matching file has been saved alongside this document as sample_mission.waypoints, ready to load into any of the three tools below.

## Three ways to send it from your laptop

### 1. QGroundControl (GUI, easiest)

- Connect Pixhawk to your laptop — USB cable, or a telemetry radio pair (SiK radio) — QGC auto-detects and connects.

- Plan view → File → Open Mission File → select sample_mission.waypoints (or a .plan).

- Review the plotted waypoints, then click Upload to send it to the vehicle.

### 2. MAVProxy (command line)

mavproxy.py --master=COM5 --baudrate 57600      # Windows: COM port; Linux/macOS: /dev/ttyUSB0 or /dev/ttyACM0

# once connected:

wp load sample_mission.waypoints

### 3. pymavlink (scripted — already a dependency of this project, pymavlink==2.4.49)

from pymavlink import mavutil, mavwp

master = mavutil.mavlink_connection('COM5', baud=57600)   # real Pixhawk over USB/telemetry

# master = mavutil.mavlink_connection('udp:127.0.0.1:14550')  # SITL instead

master.wait_heartbeat()

wp = mavwp.MAVWPLoader()

wp.load('sample_mission.waypoints')

master.mav.mission_count_send(master.target_system, master.target_component, wp.count())

for _ in range(wp.count()):

msg = master.recv_match(type=['MISSION_REQUEST', 'MISSION_REQUEST_INT'], blocking=True)

master.mav.send(wp.wp(msg.seq))

print("Mission uploaded.")

### Bridging this into your own project's pipeline

MAVSDK (what app/mavlink/px4_client.py already uses) can import a QGC file directly instead of building MissionItems by hand: drone.mission_raw.import_qgroundcontrol_mission(path) reads a .plan file and returns items ready for drone.mission_raw.upload_mission_items(). That's a legitimate way to test a hand-authored or QGC-drawn mission through your existing upload/monitor code without changing app/mission/converter.py.

## Connecting your laptop to a real Pixhawk

- USB — plug in directly; shows up as a COM port on Windows (Device Manager → Ports, e.g. COM5) or /dev/ttyACM0//dev/ttyUSB0 on Linux/macOS. Typical baud: 57600 (USB often auto-negotiates; some setups use 115200).

- Telemetry radio (SiK radio pair) — one radio on the Pixhawk, one on a USB dongle on your laptop; same COM-port style connection, standard baud 57600.

- Network (SITL only) — udp:127.0.0.1:14550; not applicable to real hardware.

## Before you expect it to arm and fly

Sending the mission successfully (upload_mission()/wp load returning success) is not the same as the vehicle accepting it for flight. Real Pixhawk hardware additionally requires, none of which SITL enforces: a real GPS fix with a settled EKF (can take tens of seconds outdoors), sensors calibrated (compass, accelerometer, level horizon), a healthy battery reading, and RC/failsafe configuration passing pre-arm checks. If arm() is refused, the specific reason is printed by the flight controller itself (visible in QGroundControl's messages panel or the MAVProxy console) — check that first before assuming the mission file itself is wrong.